TR/BR-130

Identification of aquifer parameters in Narmada basin

S. K. Singh

Abstract

The reliability of ground water models 	(predictive model/ management model) is enhanced by rapid and reasonable identification of aquifer parameter. The  important aquifer parameters are transmissivity,  T and storage coefficient, 0. Pump-test occupies a prominent position among the various methods available for  identification  of aquifer  parameters.  Graphical  methods  of identifying the aquifer parameters have been proposed by many investigators (Theis, 1935; Cooper and Jacob. 1946: and Chow, 1952) but these methods are time consuming and involve considerable subjectivity on the part of analyst. Numerical methods that use   the   objective   criteria   for   the   matching   of  theoretical   and   observed drawdown  have  been  suggested  by  McElwee(1980),  Chander  et  al.(1981), Goyal(1984) and Swamee and Ojha(1990) for constant rate pumping.

Though, several investigators	(Abu-Zeid and Scott,	1963; Abu-Zeid et al.,	1964;  and  Sternberg,	1967,	68)  have  presented  analytical  and  graphical methods for analysis of variable rate pumping test data,  only a few numerical methods for variable rate pumping, have been proposed (Butt and McElwee; 1985). For the  identification of the aquifer parameters using the drawdown observation in the pumped well,  the 'well loss'  and the 'effective radius' of pumped well need to be taken into account. Therefore, using only the observation data at the pumped  well.   instead  of	 ø, r	ø² e	(re =effective radius of well) should be determined	( Jacob:	1947).

In the  present report the  identification of T and ø by analyzing the observed drawdown in a confined aquifer due to pumping a tube-well at a variable rate  has  been attempted.  Aquifer parameters have been identified for the five test sites in Narmada basin. The main conclusions drawn from the study are;

1. A model has been developed for identification of aquifer parameters, T and ø,  utilizing  the drawdown observations  in a confined aquifer due to pumping a tube well at a variable rate. Marquardt algorithm has been used with analytical derivatives for nonlinear parameter optimization.

2.	Explicit expressions for T and	0 have been obtained  making use of the

least  square  regression.  and  the Theis equation  for constant  rate pumping.

In this method. the well loss is accounted automatically.

3.	Well  loss  affects  only  the  identification Of r	ø² e  and does  not  affect  the

identification of T.